1 x DM32 WiFi Digital Gauge Package

- DM32 WiFi Dual Channel Color Touchscreen Manometer

- USB Cable, 6' (1.8 m)

- Cat5 Cable, 7' (2 m) Blue

- DM32 USB Charging Module (120V/60Hz, 240V/50Hz, or 110V/50Hz)

- Gauge Case

- DM32 Magnetic Clip

- ISO17025 calibration of DM32

- Gauge Calibration Verification Report

1 x 6000 Series Fan Set

- 6000 Series Calibrated Fan (120V/60Hz, 240V/50Hz, or 110V/50Hz)

- Range Ring A for 5000/6000 Series Fan

- B8 Plate for 5000/6000 Series Fan

- 1 x High Power Drive for 6000 Series Fan (120V/60Hz, 240V/50Hz, or 110V/50Hz)

- Range 74,47, 29

- 8 x 4" Blank Plugs

- Power Cord

- 2 x Fan Cover

- System Calibration Verification Report

1 x Hard Panel Set for 1000/5000/6000 Series Fan

1 x Blower Door Tubing Kit

4 x Roll of Grill Masks

1 x Umbilical for 1000/5000/6000 Series Fan, 7 ft (2 m)

1 x Large Frame Package

1 x Fan Case

1 x Accessory Case

| Specifications below are for each individual fan. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fan Model #: | 6000 |  |  |  |  |
|  | Flow Into: | CFM | litres/sec | m3/h |  |
| Maximum Flow*, per fan | Free Air | 8,100 | 3,823 | 13,762 |  |
|  | 50 Pa | 7,700 | 3,634 | 13,082 |  |
|  | 75 Pa | 7,400 | 3,492 | 12,573 |  |
| Minimum Flow: on Range 29 | 50 Pa | 9 | 4 | 15 |  |
| Dimensions: | 26" × 25.5" × 9.5" (66 x 65 x 24 cm) |  |  |  |  |
| Weight: | 36.3 lb (16.5 kg) or 38 lb (17.2 kg) with Ring A & Plated B8 installed |  |  |  |  |
| Flow Accuracy: | +/-5% |  |  |  |  |
| Power: | 120V@60hz/22.0A or 240V@50hz/10.9A or 110V@50Hz/22.0A |  |  |  |  |
| Recommended Fan Calibration Interval: | 5 years |  |  |  |  |